PROJECT:

NEONFLEX PROX-V SERIES

LOCATION:

CATALOG NUMBER:

TYPE:

24V DC Pixel RGBW Top Bend Neon

Neonflex Pro APEX is the ultimate solution for achieving seamless lines of direct view lighting for any architectural-grade install. UV, flame, and saltwater resistant, Neonflex Pro APEX is not only durable, but easy to install.

- Improved construction includes PVC backer to prevent hot-spots and enhance durability

- RGBW vertical (top bend) LED neon

- Lumen output performance up to 145Lm per foot

- Consumes up to 4.5W per foot

- Cuttable every 5" with 24ft max run

- UV, flame, and saltwater resistant

- IP67 rating featuring sealed silicone jacketing

- Dimmable (5-100%) with most OMX controls

- cULus Listed for indoor and outdoor wet locations

- 54,000 hours rated life

## c@us

### LISTED

NEONFLEX PROX-V PIXEL RGBW SERIES QUICK SPECS

#### VOLTAGE WATTAGE LUMENS CCTOPTIONS CRI

##### 24VDC

4.5W/ft

<':24CM

i

Up to 145Lm/ft RGBW (3000K) 80+

##### 2 PERSON INSTALLATION RECOMMENDED MINIMUM BEND RADIUS ;,24CM

CUTTING POINTS	5" (50.8mm)

MAX RUN	24ft (7.3m)

IP RATING	IP67 - Jacketed

PROTOCOL	DMX IC: UCS512CL

RESOLUTION	5" (127mm) pixel resolution

#### DIMMING DIMENSIONS

##### DMX

0.63" (16mm) W x 0.67" (17mm) H

Lighted Surface

✓

¢=,

BEAM ANGLE	120°

STORAGE TEMP	 -25°C (-13°F) to 60°C (140°F)" AMBIENT TEMP	-25'C (-13°F) to 45'C (113°F)" OPERATING TEMP	 -25°C (-13°F) to 45°C (113'F)"

CERTIFICATIONS	cULus Listed - Indoor/Outdoor Wet Location

RATED LIFE	54,000 Hours

**Do not install in inclement weather or extreme temperature conditions (i.e. -25°C (-13°F) and below). Recommended to order Custom Cut direct from American Lighting for outdoor installations.

NEONFLEX PROX-V PIXEL RGBW SERIES QUICK LOOK

36" 20 AWG	Molded Connection

0.63" (16mm) W x 0.63" (17mm) H

q..	j	•---�

=

28ft Max Run

NEONFLEX PROX-V SERIES CUSTOM CUT ORDERING	*Custom Cut Orders - Please Allow Up To 2-4 Weeks Order Processing & Delivery*

�I----�

SERIES	COLOR	LENGTH

�-Pz_M_-_N_F_P_R_o_x_-v_�I

PXLRGBW = Pixel RGB+3000K

Custom Cuts:	Specify desired length according to cut increment in INCHES. (See page 4 for cut increment chart). Assembly Fee per length of NeonFlex Pro-V Series Custom Cut. Custom orders are deemed Final Sale and are not able to be returned or refunded.

NEONFLEX PROX-V SERIES ASSEMBLY FEES

ITEM NUMBER

CC-LF-NFPROX-V-5-F CC-LF-NFPROX-V-5-SL

DESCRIPTION

Right Side

+	Front cable entry

FrootcabH!entry

Sioecableentry

Sidecableeotry

Bonomcableentry

Lead: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Front Lead: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Side Left

CC-LF-NFPROX-V-5-SR	Lead: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Side Right

CC-LF-NFPROX-V-5-B	Lead: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Bottom

CC-LTF-NFPROX-V-5-F	Lead & Tail: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Front

CC-LTF-NFPROX-V-5-SL  Lead & Tail: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Side Left CC-LTF-NFPROX-V-5-SR  Lead & Tail: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Side Right CC-LTF-NFPROX-V-5-B	 Lead & Tail: 10ft Cable Conkit, Solder, and Injection Mold Seal for Top Bend 2Pin Bottom CC-SETUP-NFPRO	1-9 CUT SMALL ORDER SETUP FEE FOR NFPRO

NEONFLEX PROX-V SERIES SPECIFICATION DETAILS

| ITEM NUMBER | VOLTAGE | CCT | LENGTH | LUMENS / FT | WATTAGE/FT | IP RATING | CUTTING | MAX RUN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PZM-NFPROX-V-PXLRGBW-CC | 24VDC | RGB+3000K | Custom Cut | 145Lm/ft | 4.5W/ft | IP67 | 5" (50.1mm) | 24ft |

NEONFLEX PROX-V SERIES ACCESSORIES

##### ITEM NUMBER	DESCRIPTION

PZM-NFPROX-CHANG-1M	NeonFlex Pro Apex 1M Aluminum Extrusion/Internal Gear

PZM-NFPROX-CHANFLEX-1M	NeonFlex Pro Apex 1M Flex Stainless Steel Channel

PZM-NFPROX-CHANMl1FLEX-1M	NeonFlex Pro Apex 1M Single Flange Flexible Mud-in Alumi-

num Channel

PZM-NFPROX-CHANMl2FLEX-1M  NeonFlex Pro Apex 1M Double Flange Flexible Mud-in Alumi-

num Channel

'

0.67"

0

m

0

o_

"1

a) t-, 0

2.32"

�

CHANG

CHANMl2FLEX

PZM-NFPROX-CLIPSAL PZM-NFPROX-CLIPSPC

NeonFlex Pro Apex Aluminum Mounting Clips NeonFlex Pro Apex PC Mounting Clips

CHANFLEX

CHANMl1FLEX

0.79"

i'o

t-:

0

1.54"

CLIPSAL	CLIPSPC

RECOMMENDED  POWER SUPPLIES	*Phase Dimming Compatible With Most MLV, ELV, and TRIAC Dimmers*

| ITEM NUMBER | DESCRIPTION | MAX/ MIN LOAD | INPUT VOLTAGE | OUTPUT VOLTAGE | DIMMABLE | DIMENSIONS |
| --- | --- | --- | --- | --- | --- | --- |
| PZM-DRJADPT24V-30 | Adaptive Series 30W | 30W/3W | 100-277V AC | 24V DC | Yes' | 6.49"L x 3.6"W x 1.05"H |
| PZM-DRJADPT24V-60 | Adaptive Series 60W | 60W/6W | 100-277V AC | 24V DC | Yes* | 7.4"L x 3.6"W x 1.05"H |
| PZM-DRJADPT24V-96 | Adaptive Series 96W | 96W /9.6W | 100-277V AC | 24V DC | Yes' | 8.66"L x 3.6"W x 1.61"H |
| PZM-DRJADPT24V-192 | Adaptive Series 192W (2 x 96W) | 192W/19.2W | 100-277V AC | 24V DC | Yes* | 10.94"L x 4.25"W x 1.B"H |
| PZM-DRJADPT24V-288 | Adaptive Series 288W (3 x 96W) | 288W /28.BW | 100-277V AC | 24V DC | Yes* | 11.85"L x 4.25"W x 1.8"H |